# Gondwana Phase 1 Deployment Pack

## Ecological Boundary Pilot – 4.2 km Radius (Centred on 81 Boscombe Road, Brookfield)

### Purpose (what Emergent is building)

Implement the Biodiversity Performance & Prioritisation Framework (BPPF) as a repeatable, spatially explicit decision engine to:

- define a defensible baseline,

- prioritise protection and restoration,

- quantify ecological risk,

- and measure uplift over time.

Biodiversity Performance & Prio…

### Scope

- Boundary: 4.2 km radius ecological study area (not 10 km)

- Primary signal: landscape + vegetation (satellite + official datasets)

- Analytical unit: 1 km × 1 km grid cells (“Performance Units”)

Biodiversity Performance & Prio…

## Pack A — Boundary & GIS Definition

### A1. Boundary definition (must be unambiguous)

- Centre point: 81 Boscombe Road, Brookfield

- Radius: 4.2 km

- Projection: GDA2020

- Outputs:

  - Boundary polygon (GeoJSON + Shapefile)

  - Basemap PDF (for human reference)

### A2. Property layer (optional but recommended)

- Parcel boundaries + RP numbers included where available (as an operational layer, not the analytical unit)

### A3. Required ecological layers (dataset versions logged)

- Queensland Regional Ecosystem (RE) mapping (and REDD remnant definition)

- Waterway / drainage layers

- Protected areas layer

- Elevation (DEM)

- Any state land-use layer available

Important: No RE codes are to be assumed in advance. The RE list must be extracted inside the 4.2 km boundary and recorded.

## Pack B — BPPF Grid Implementation (Core)

### B1. Create the grid

- Overlay 1 km × 1 km grid across the 4.2 km boundary

- Clip grid to boundary

- Assign unique Grid_ID to each cell

Biodiversity Performance & Prio…

### B2. Baseline definition (audit-ready)

- Baseline is defined using:

  - RE mapping + REDD remnant definition

  - Conservation status classes (Endangered / Of Concern / Least Concern)

  - Baseline date + dataset versions recorded

Biodiversity Performance & Prio…

## Pack C — Scoring Engine (Priority Index 0–100)

Use the BPPF weighting structure exactly (Version 1.0):

- Ecological Value — 40%

- Connectivity — 20%

- Conservation Sensitivity — 20%

- Uplift Potential — 20%

Biodiversity Performance & Prio…

Priority Score formula:
Priority Score = (EV × 0.4) + (CON × 0.2) + (SENS × 0.2) + (UP × 0.2)

Biodiversity Performance & Prio…

### C1. Classification bands (must match framework)

- 80–100 Critical Biodiversity Core

- 65–79 Strategic Protection Zone

- 50–64 High Uplift Zone

- 35–49 Connectivity Support

- <35 Low Ecological Leverage

Biodiversity Performance & Prio…

### C2. Output requirements

- Grid table export (CSV): Grid_ID + dimension scores + total + category

- GIS heatmap layer (GeoJSON/Shapefile)

- Area breakdown by category

## Pack D — Vegetation Baseline (Satellite-First)

### D1. NDVI and vegetation condition layers

- Produce NDVI baseline and trend layers suitable for overlay against the grid

- Log:

  - sensor/source (e.g., Sentinel-2 if used)

  - cloud masking approach

  - baseline window (e.g., trailing 12 months)

### D2. What NDVI is used for in Phase 1

- As a repeatable vegetation vigour proxy to support:

  - uplift potential scoring,

  - degradation identification,

  - restoration opportunity detection.

(We can be explicit that NDVI is a proxy and not a direct biodiversity measure — that’s part of defensibility.)

## Pack E — Connectivity & Corridor Logic (Ecological Boundary Justification)

This is where we justify why 4.2 km is ecological, not arbitrary.

### Insert this exact text into the pack (ready to use)

Ecological Boundary Rationale (4.2 km):
“The Phase 1 boundary is defined as a 4.2 km radius to capture the functional ecological system surrounding the Gondwana anchor site, including local hydrological influence, remnant patch interactions, and short-range connectivity dynamics. This radius is intended to include the immediate corridor and edge-context that drives habitat integrity, fragmentation pressure, and restoration leverage, while remaining small enough to enable high-resolution auditability and repeatable monitoring using a fixed 1 km² performance grid.”

Biodiversity Performance & Prio…

Regional Biodiversity Priority …

(That reads like a methods section, which is what you want.)

## Pack F — Deliverables to Hand Back (What Emergent must deliver)

- Boundary package (4.2 km)

- 1 km² grid package (Performance Units)

- Dataset register (source + version + date)

- BPPF scores per grid cell + category

- Priority heatmap GIS layer + PDF visual

- Ranked engagement list (grid clusters → properties)

- Short “Methods + Assumptions” appendix (audit-ready)

All consistent with the BPPF and heatmap methodology structure.

Biodiversity Performance & Prio…

Regional Biodiversity Priority …

## What I’m correcting from the earlier draft

- No acoustics in Phase 1 (unless later added as an overlay in monitoring)

- No pre-listed RE codes until extracted from the 4.2 km boundary

- No 10 km references anywhere